Title: Serial Configuration 3D Printer

(Group Project (Group no. 10))

Computer-Integrated Manufacturing

Project Report

by

Aayush Vats (25263101)

Akash Gaurav (24263201)

Amit Kumar Anand (25263103)

Anirudha Pratihar (2206303)

Instructor

Dr. Keval S. Ramani

School of Mechanical Sciences

Indian Institute of Technology Goa

## Table of Contents:

- Introduction

  - Introduction

  - Objective

  - Motivation

  - Problem Statement

- Methodology and description

  - Mechanical system design

  - Electronics Part Function

  - Control

- Applications

3.1 Engineering Prototyping and Product Development

3.2 Educational and research applications

3.3 Small-scale production and Manufacturing

3.4 Robotics and Automation

Worklog

# Introduction:

## 1.1 Introduction

Additive manufacturing is one of the most common techniques in modern engineering, which enables rapid prototyping, cost-effective small-scale production. To this end, we use a Cartesian-based system, which is widely used due to its simplicity, precision, and reliability. The printer operates on 3 mutually orthogonal axes, X, Y, and Z, and it allows the print-head accordingly for a defined workspace. This project is pointing towards the design of a 3D printer using 20x20 aluminium extrusion, linear guides, GT pulley belts, and a ball screw for z-axis motion. For running the overall setup and its functioning, we use 4 stepper motors, and these motors are controlled by the microcontroller Arduino Mega 2560. The printing platform is built using an Aluminium sheet, a heat pad, and an extrusion mechanism, resulting in the creation of a reliable and efficient 3D printer for applications of prototyping and academic use.

## 1.2 Objective:

The key objective for this project to construct a 3D printer that produces precise and high quality prints. Expected characters to be achieved are:

- To construct a rigid frame by using aluminum extrusions and linear guides.

- Obtaining accurate movements of all axes using Belts(GT-2) , ball screw and stepper motors.

- Filament flow from the extruder should be uniform and maintain a steady state.

- Power management should be taken into account for building control system using Arduino Mega 2560.

- To create an overall cost-effective Printer.

## 1.3 Motivation:

3D printing has many applications for designing any part, as many commercial machines are costly (especially high-performance ones). This project is designed to create a printer with custom features that is more affordable and adaptable for improved features. The hands-on process for building it from scratch and the use of various principles from different disciplines of mechanical, electronics, and software engineering. This project is not limited to technical or problem-solving skills, but also results in building a versatile platform and can be upgraded with more and new features.

## 1.4 Problem Statement:

The gap between low-cost models (that suffer from poor quality and inflexibility) and high-performance models, which are too costly for project purposes. This project fulfills the need for a reliable, mid-range printer which is both good in precision and lies within an affordable budget. The main challenge is to integrate robust mechanical, electronic and software components into a cohesive, high-performing and building an safe system.

# Methodology and Description

## 2.1 Mechanical System Design

The mechanical system is the basis of the overall printing mechanism that decides work volume and provides motion and guides for performing the printing task. The details are discussed as follows.

2.1.1 Frame Structure:

The basis of the structure is built with 2020 aluminum extrusions because of its high strength-to-weight ratio and rigidity. These profiles are assembled into a rectangular frame using precise corner brackets and fasteners. To this end, we prioritize stability and resistant to vibrations, that sets the foundation of all other components.

2.1.2 Motion Transmission System:

The printer will use two types of systems for 2 different purposes:

X and Y-axes: use of GT2 belts and pulleys driven by NEMA stepper motors. This configuration will provide position and movement to the print head and bed.

Z-axes: For these axes, we used a 1mm ball pitch screw mechanism connected to a stepper motor via a flexible coupler. This provides vertical movement and less backlash, that maintains consistent height layer throughout the printing process.

2.1.3 Extrusion system:

The printer utilizes an MK8-style extrusion system, which integrates filament feeding and melting components into the printhead assembly mounted on the x-axis carriage. The specifications are

- Standard extrusion brass nozzle 0.4mm.

- A compatible design to use 1.75mm diameter Filament .

- Extrusion capacity approximately up to 24cc/hour.

- Heating system is designed that incorporates ceramic heating insulation for thermal stability and energy efficiency.

- Direct driver transmission that provides consistent filament pushing force with minimal weight.

- Temperature sensing and Cooling system: 100K NTC thermistor is used and integrated cooling fan is used for heat dissipation

2.1.4 Build Platform:

The build platform consists of an aluminium sheet mounted on Y-axis, which provides a flat surface for print adhesion and heat distribution. Aluminium material ensures dimensional stability and even distribution of heat across the overall area.

## 2.2 Electronics parts functions:

The electronic system is considered as nervous system of the 3D printer, which integrates power distribution, motor control, user interface, and sensory feedback into a cohesive operational unit.

2.2.1 Motor Connections :

For the system usage of multiple of NEMA 17 stepper motors for mechanical control    with precision:

Three-axis positioning (x,y,z) for accurate movement control.

Dedicated extruder drive for consistent filament feeding.

Power management: The Motor receives controlled power through the RAMPS 1.4 shield that ensures synchronized movement and precise monitoring.

2.2.2 User Interface and Control Systems:

Operator interaction is done by the display system and its features are LCD display with monitoring status, a Rotary encoder with push button for seamless navigation and adjustment parameters.

Integrated LED workspace with proper dedicated switches.

Visual status indicators for power and operational states.

2.2.3 Sensors (Thermistors)

100K NTC thermistor arrays in the hotend and heated bed for temperature sensing. PID-controlled heating systems that maintain operating temperatures at optimal states.

Thermal runaway protection ensures operational safety.

Mechanical end-stop switches for homing and position calibration.

2.2.4 Integration of systems:

All subsystems are interconnected and operated by the overall system, which creates an operational environment that synchronizes temperatures, positions, and interaction with the user. These all parts are integrated to produce precise control with reliable performance throughout the cycle.

## 2.3 Controls:

We have been using the widely known Marlin firmware which many of the 3D printers around the world use. This firmware comes up with the option to modify the program according to our printers configuration.  The most important aspect of this setup is that we use PID controls for both motor motions and temperature control in both bed and Extruder . A code file has been attached with the folder.

# Applications:

Making of Cartesian based 3D printer has a large range of applications includes many engineering disciplines, industrial, scientific and educational field of domains. 3D printers has ability to fabricate complex geometries and shapes, designs and with needed customizable parameters. Some examples where use of 3D printers are done as follows:

3.1 Engineering Prototyping and Product development:

By application of 3D printer, we can convert CAD models into physical models in certain types (few hours). Cartesian 3D printer enables students and users to

- Evaluate form, fit and functionality of design prototypes

- Test of mechanical assemblies before manufacturing in large scale

- To validating design concepts through physical models

- To iterate product designs quickly

Mechanical parts can also be produced for testing. Other applications are robotic prototypes, concept realization in industries such as automotive, consumer products, and electronics.

3.2 Educational and research applications:

In disciplines where the study of mechanical design, mechatronics, electronics, or automation ability to fabricate real products.

- Academic concept involves mechanical concepts such as kinematics, motion control, tolerance design, etc.

- Electronic students explore power management, microcontroller-based control in the setup.

- Research scholars can make their setups accordingly.

3.3 Small-scale production and Manufacturing:

Modern 3D printing has found significant application in small scale and coustomized production. The printer can manufacture:

- Jigs, fixtures and alignment tools

- Spare parts or replacements for machine components.

- Low-volume specialized components for industries

- Enclosures for IoT devices, sensors and electronics.

Without investing in large-scale manufacturing equipments

3.4 Robotics and Automation :

In robotics domain, 3D printing is widely used for robotics development, and to custom robot components.

Hence, we can use a Cartesian-based printer to fabricate:

- Robot arms, chassis.

- Gear Systems and transmission housings.

- Structural parts for drones or mobile robots (which are lightweight).

Work contributions:

Aayush Vats: Structural and Electronics assembly and Code part (Marlin Software),

Suggestions for electronic parts functions in report writing and presentation.

Akash Gaurav: Structural Assembly, fitting with screws, nuts and bolts, filling, cutting, axes and extrusion motors setup and 3D printing of structural parts

Amit Kumar Anand: Presentation format, Assisting for frame assembly.

Buying aluminium strip and aluminium bed from the market, Report 3 unit and presentation

Anirudha Pratihar: Worked on BOM of parts, Filing of parts, and the Report of the First 2 units